Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#04)

Use of Venus Simulator for Register Level Execution of RISC-V Arithmetic and Memory Instructions

Student Name: __Syed Sikandar Abbas_ Roll No: _24I-6553____ Section: CE-B

Date performed: _September 9, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

In this task we were required to perform arithmetic operations on registers using assembly language and simulate it in Venus Simulator. Values of A and B were initially specified and A+B, A-B, A*B, A^2 operations were to be performed. Along with it, we were also required to express the machine representation of each instruction in decimal.

Code:

.text .data .globl main addi s0, x0, 5 # A addi s1, x0, 4 # B add s2, s0, s1 # C = A + B sub s3, s0, s1 # D = A - B mul s4, s0, s1 # E = A * B mul s5, s0, s0 # F = A ^ 2 li a7, 93 li a0, 0 ecall

Output:

Part V:

Instruction # 1: addi s0, x0, 5 Hex = 0x500413 Decimal = (3*16^0) + (1*16^1) + (4*16^2) + (5*16^5) = 5243923 Instruction # 2: addi s1, x0, 4 Hex = 0x400493 Decimal = (3*16^0) + (9*16^1) + (4*16^2) + (4*16^5) = 4195475 Instruction # 3: add s2, s0, s1 Hex = 0x940933 Decimal = (3*16^0) + (3*16^1) + (9*16^2) + (4*16^4) + (9**16^5) = 9699635 Instruction # 4: sub s3, s0, s1 Hex = 0x409409B3 Decimal = (3*16^0) + (11*16^1) + (9*16^2) + (4*16^4) + (9*16^5) + (4*16^7) = 1083443635 Instruction # 5: mul s4, s0, s1 Hex = 0x2940A33 Decimal = (3*16^0) + (3*16^1) + (10*16^2) + (4*16^4) + (9*16^5) + (2*16^6) = 43256371 Instruction # 6: mul s5, s0, s0 Hex = 0x2840AB3 Decimal = (3*16^0) + (11*16^1) + (10*16^2) + (4*16^4) + (8*16^5) + (2*16^6) = 42209971 Instruction # 7: li a7, 93 Hex = 0x5D00893 Decimal = (3*16^0) + (9*16^1) + (8*16^2) + (13*16^4) + (5**16^5) = 97523859 Instruction # 8: li a0, 0 Hex = 0x513 Decimal = (3*16^0) + (1*16^1) + (5*16^2) = 1299 Instruction # 9: ecall Hex = 0x73 Decimal = (3*16^0) + (7*16^1) = 115

Task 2:

Description:

In this task we were required to use the values of A and B, along with the previous task’s outputs (C,D, E, F) and combine them in a single expression to produce m as m = (A – C) + (D + E) * F.

Code:

.text .data .globl main addi s0, x0, 5 # A addi s1, x0, 4 # B add s2, s0, s1 # C = A + B sub s3, s0, s1 # D = A - B mul s4, s0, s1 # E = A * B mul s5, s0, s0 # F = A ^ 2 sub s6, s0, s2 # A - C add s7, s3, s4 # D + E mul s8, s7, s5 # (D+E)*F add s9, s6, s8 # m li a7, 93 li a0, 0 ecall

Output:

Task 3:

Description:

In this code we were required to store hexadecimal values 0xFFF and 0xF0F in two separate registers and perform AND, OR and XOR operations on those registers.

Code:

.text .data .globl main li s0, 0xFFF li s1, 0xF0F and s2, s0, s1 or s3, s0, s1 xor s4, s0, s1 li a7, 93 li a0, 0 ecall

Output:

Task 4:

Description:

In this code we were given an assembly language code along with its C equivalent code. We were supposed to find out which function is being performed by the code and correct any issues in that Assembly code. The C code was given for reference. - The assembly code was supposed to be adding the contents of an array and finally produce the sum of all the elements of the given array. - It had .section which is not supported in Venus. It had .global but Venus supports .globl. These issues were corrected. There was no main function hence there was no array or its value. The assembler was working on garbage values and was producing unexpected outputs. So a .globl main was declared which declared stored the array address and its size and passed it onto the arraysum function. - There were two labels each named ‘1’ and, 1b and 1f were used for branching which created ambiguity for the assembler when the .globl main was defined. These labels were changed to for and end to create seamless branching. After all these changes, the code worked fine.

Code:

.data array: .word 10, 20, 30, 40, 50 .text .globl main .globl arraysum main: la a0, array # a0 = pointer to array li a1, 5 # a1 = length of array jal ra, arraysum # Result returns in a0 (150) li a7, 10 # ecall 10 = clean exit ecall arraysum: # a0 = int a[] # a1 = int size # t0 = ret # t1 = i li t0, 0 # ret = 0 li t1, 0 # i = 0 for: # For loop bge t1, a1, end # if i >= size, break slli t2, t1, 2 # Multiply i by 4 (1 << 2 = 4) add t2, a0, t2 # Update memory address lw t2, 0(t2) # Dereference address to get integer add t0, t0, t2 # Add integer value to ret addi t1, t1, 1 # Increment the iterator j for # Jump back to start of loop (1 backwards) end: mv a0, t0 # Move t0 (ret) into a0 ret # Return via return address register

Output:

Task 5:

Description:

In this task we were supposed to store the table of two in a data type .word and store them in different registers. It declared table: .word 2, 4, 6, … , 18, 20 in .data section. Then it called the main which loads the address of the table’s first element in a1 register using la (load address) command. Then it repeatedly used lw (load word) command to load elements of the array table into saved registers from s0 to s9. Indexing was used in this code in this format: lw dest_reg, offset(base_address).

Code:

.data table: .word 2, 4, 6, 8, 10, 12, 14, 16, 18, 20 .text .globl main main: la a1, table lw s0, 0(a1) lw s1, 4(a1) lw s2, 8(a1) lw s3, 12(a1) lw s4, 16(a1) lw s5, 20(a1) lw s6, 24(a1) lw s7, 28(a1) lw s8, 32(a1) lw s9, 36(a1) li a7, 93 li a0, 10 ecall

Output: