Computer Architecture Lab

(EL-2012)

LABORATORY MANUAL

Fall 2026

(Lab#05)

Use of Venus Simulator for Register Level Execution of RISC-V Control Instructions

Student Name: __________ Roll No: ________________ Section: _A__

Date performed: _____________, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

____________________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad | Version: 2.0 |
| --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah | Updated: Fall 2026 |
| Learning Objectives: - To use Venus simulator for simulation of RISC-V assembly language program - To monitor the register contents after execution of RISC-V assembly instructions - To execute load word, store word, loop and nested loops using Venus simulator Tool Used: |  |  |  |  |
| Venus Simulator (available online) Assembly Language: Refer to earlier Lab#01 “Use of Venus Simulator for Register Level Execution of RISC-V Instructions” for background information on Venus simulator and its usage, specially: - RISC-V Registers - Assembly Instructions - Data Declarations - Load / Store Instructions - Indirect and Based Addressing RISC-V Register File: ### Control Instructions Comparison for conditional branches is built into the instructions themselves. Branches type target # conditional branch to program label target beq t0, t1, target # branch to target if t0 = t1 blt t0, t1, target # branch to target if t0 < t1 ble t0, t1, target # branch to target if t0 <= t1 bgt t0, t1, target # branch to target if t0 > t1 bge t0, t1, target # branch to target if t0 >= t1 bne t0, t1, target # branch to target if t0 <> t1 Table 1 Jumps j target # unconditional jump to target jal ra, target # unconditional jump to PC+target and save PC in ra jalr zero, const(ra) # unconditional jump to ra+const and save PC in zero Table 2 ### Using Venus Simulator: Where to Get Venus simulator? This is an online simulator and may be accessed at: venus (thaumicmekanism.github.io) - If you wish to download a copy to run locally on your laptop, get it from here: https://courses.missouristate.edu/KenVollmar/mars/download.htm Procedure: - When the Venus simulator opens in your browser window you can either type or copy/paste your code in the Editor window. - Once done (or anytime you want to) assemble and simulate your code by moving to the Simulator tab and clicking on the “Assemble & Simulate from Editor” button. - Once the code is assembled, you may Run, Debug or Trace execution of your code. - The Memory and Register or Cache state, will be seen in real time on the right side of the browser window. - There are a variety of options and features to explore in Venus online simulator, explore, experiment and exercise these on your own. - Try the three example codes below to learn about arithmetic, memory, and array manipulation in RISC-V. - When working through the code examples, you could print out or access this RISC-V reference sheet to help you: James Zhu’s RISC-V Reference. ### Example Codes: Example 1: Decide outcome based on two numbers blt s0, s1, label #jump to label if s0 < s1 label: #execute code if s0 < s1 #true clause #execute code if s0 >= s1 #false clause For more RISC-V assembly programs, please visit: https://marz.utk.edu/my-courses/cosc230/book/example-risc-v-assembly-programs |  |  |  |  |

Tasks:

- Indicate which of the assembly instructions listed in Table 1 and Table 2 above are actual instructions and which are pseudo-instructions? How do you know?

- Write a code snippet with just two instructions beq and j (conditional branch and jump). Simulate to understand the difference between them. Explain.

- Run the codes given below, comment each code line by line, and explain what it does:

Code 1:

addi	s0,	s0,	4

addi	s1,	s1,	6

beq	s0,	s1,	Fin

True:

add	s2,	s1,	s0

j	end

Fin:

sub	s2,	s1,	s0

end:

Explanation:

Code 2:

addi	s0,	s0,	4

addi	s1,	s1,	4

beq	s0,	s1,	Fin

True:

add	s2,	s1,	s0

j	end

Fin:

sub	s2,	s1,	s0

End:

Explanation:

- Convert given C code into RISC-V assembly and simulate using Venus simulator. Consider a=5, b=10, d=4, e=3.

if (a = = b)

c =  d – e ;

else

c = d + e ;

- Write RISC-V assembly code to shift a number (e.g. 2) to the left twice and simulate using Venus simulator.

- Write RISC-V assembly language for given loop and simulate it using Venus.

Initialize counter = 0;

for i = 1 to 5

increment counter;

end

- Write RISC-V assembly language for given loop and simulate it using Venus.

int num[10];

void main(){

i=0;

while (i<5)

num[i] = i+5;

}

- Write RISC-V assembly code for nested loops and simulate it using Venus.

Counter=0;

for (int x=0; x<3; x++) {

for (int y=0; y<2; y++) {

Counter=x+y;

}

}