Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#07)

Implementation of Arithmetic Logic Unit (ALU)

Student Name: __Syed Sikandar Abbas________ Roll No: _24i-6553_______________ Section: CE-B

Date performed: _6/10/, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

Using the given design, write a Verilog code at all level of abstraction to make a Module for 1 bit ALU having inputs (1 bit) as A and B as input data, carry_in, select line (2 bit input) to select between different operations i.e. AND, OR and ADD/SUB. The module will have an Output for Result of operations and another output for carry_out. Selection between ADD/SUB will be provided by another input b_inv. Show simulation results for every instruction executed. Make sure to design a circuit diagram using logisim and a truth table to verify the working of ALU.

Code:

Gate Level: module myAND( output O, input A, B, C, D ); wire t0, t1; and(t0, A, B); and(t1, C, D); and(O, t0, t1); endmodule module Multiplexer_GLevel( output O, input S0, S1, I0, I1, I2, I3 ); wire SN0, SN1; not(SN0, S0); not(SN1, S1); wire t0, t1, t2, t3; myAND A1(t0, I0, SN0, SN1, 1); myAND A2(t1, I1, SN0, S1, 1); myAND A3(t2, I2, S0, SN1, 1); myAND A4(t3, I3, S0, S1, 1); wire to0, to1; or(to0, t0, t1); or(to1, to0, t2); or(O, t3, to1); //or'ed 3 inputs using 3 2-input modules endmodule module HalfAdder_GLevel(Sum, Carry, A, B); input A, B; output Sum, Carry; xor sum(Sum, A, B); and carry(Carry, A, B); endmodule module FullAdder_GLevel(Sum, Carry, A, B, Cin); input A, B, Cin; output Sum, Carry; wire s1, c1, c2; HalfAdder_GLevel ha1(s1, c1, A, B); HalfAdder_GLevel ha2(Sum, c2, s1, Cin); or carry(Carry, c1, c2); endmodule // -----------MAIN MODULE------------------ module ALU_GLevel(A, B, carry_in, result, carry_out, S0, S1, b_inv); input A, B, S0, S1, b_inv, carry_in; output result, carry_out; wire b_final; wire fa_sum; wire fa_carry; wire And; wire Or; xor (b_final, B, b_inv); //b final will be used in the circuit FullAdder_GLevel fa1(fa_sum, fa_carry, A, b_final, carry_in); and a(And, A, B); or o(Or, A, B); /* S0 S1 0 0 - AND 0 1 - OR 1 0 - ADD/SUB 1 1 - ADD/SUB */ Multiplexer_GLevel mux(result, S0, S1, And, Or, fa_sum, fa_sum); assign carry_out = fa_carry; endmodule module _1bitALU_tb; reg A, B, Cin; reg b_inv; reg S0, S1; wire O, Cout; ALU_GLevel myALU(A, B, Cin, O, Cout, S0, S1, b_inv); initial begin A = 0; B = 0; Cin = 0; b_inv = 0; S0 = 0; S1 = 0; //0 AND 0 = 0 #10; A = 1; B = 1; S0 = 0; S1 = 0; b_inv = 0; //1 AND 1 = 1 #10; A = 0; B = 1; S0 = 0; S1 = 1; b_inv = 0; //0 OR 1 = 1 #10; A = 1; B = 1; S0 = 0; S1 = 1; b_inv = 0; //1 OR 1 = 1 #10; A = 1; B = 1; Cin = 0; S0 = 1; S1 = 0; b_inv = 0; //1 + 1 = 10 #10; A = 1; B = 1; Cin = 1; S0 = 1; S1 = 0; b_inv = 0; //1 + 1 + 1 = 11 #10; A = 1; B = 0; Cin = 1; S0 = 1; S1 = 0; b_inv = 1; //1 - 0 + 1 = 1 + 1 + 1 = 11 #10; A = 0; B = 1; Cin = 1; S0 = 1; S1 = 1; b_inv = 1; //0 - 1 + 1 = 0 + 1 + 1 = 10 end endmodule DataFlow Level: module myAND( output O, input A, B, C, D ); assign O = A & B & C & D; endmodule module Multiplexer_Dataflow( output O, input S0, S1, I0, I1, I2, I3 ); assign O = (~S1 & ~S0 & I0) ¦ (~S1 & S0 & I1) ¦ (S1 & ~S0 & I2) ¦ (S1 & S0 & I3); endmodule module HalfAdder_Dataflow(Sum, Carry, A, B); input A, B; output Sum, Carry; assign Sum = A ^ B; assign Carry = A & B; endmodule module FullAdder_Dataflow(Sum, Carry, A, B, Cin); input A, B, Cin; output Sum, Carry; assign Sum = A ^ B ^ Cin; assign Carry = (A & B) ¦ (B & Cin) ¦ (A & Cin); endmodule // -----------MAIN MODULE------------------ module ALU_Dataflow(A, B, carry_in, result, carry_out, S0, S1, b_inv); input A, B, S0, S1, b_inv, carry_in; output result, carry_out; wire b_final; wire fa_sum; wire fa_carry; wire And; wire Or; assign b_final = B ^ b_inv; assign {fa_carry, fa_sum} = A + b_final + carry_in; assign And = A & B; assign Or = A ¦ B; assign result = ({S1, S0} == 2'b00) ? And : ({S1, S0} == 2'b01) ? Or : fa_sum; assign carry_out = fa_carry; endmodule module _1bitALU_tb; reg A, B, Cin; reg b_inv; reg S0, S1; wire O, Cout; ALU_Dataflow myALU(A, B, Cin, O, Cout, S0, S1, b_inv); initial begin A = 0; B = 0; Cin = 0; b_inv = 0; S0 = 0; S1 = 0; #10; A = 1; B = 1; S0 = 0; S1 = 0; b_inv = 0; #10; A = 0; B = 1; S0 = 0; S1 = 1; b_inv = 0; #10; A = 1; B = 1; S0 = 0; S1 = 1; b_inv = 0; #10; A = 1; B = 1; Cin = 0; S0 = 1; S1 = 0; b_inv = 0; #10; A = 1; B = 1; Cin = 1; S0 = 1; S1 = 0; b_inv = 0; #10; A = 1; B = 0; Cin = 1; S0 = 1; S1 = 0; b_inv = 1; #10; A = 0; B = 1; Cin = 1; S0 = 1; S1 = 1; b_inv = 1; end endmodule Behavioural Level: module myAND( output reg O, input A, B, C, D ); always @(*) begin O = A & B & C & D; end endmodule module Multiplexer_Behavioral( output reg O, input S0, S1, I0, I1, I2, I3 ); always @(*) begin case ({S1, S0}) 2'b00: O = I0; 2'b01: O = I1; 2'b10: O = I2; 2'b11: O = I3; default: O = 1'b0; endcase end endmodule module HalfAdder_Behavioral(Sum, Carry, A, B); input A, B; output reg Sum, Carry; always @(*) begin Sum = A ^ B; Carry = A & B; end endmodule module FullAdder_Behavioral(Sum, Carry, A, B, Cin); input A, B, Cin; output reg Sum, Carry; always @(*) begin {Carry, Sum} = A + B + Cin; end endmodule // -----------MAIN MODULE------------------ module ALU_Behavioral(A, B, carry_in, result, carry_out, S0, S1, b_inv); input A, B, S0, S1, b_inv, carry_in; output reg result, carry_out; reg b_final; reg fa_sum; reg fa_carry; reg And; reg Or; always @(*) begin b_final = B ^ b_inv; {fa_carry, fa_sum} = A + b_final + carry_in; And = A & B; Or = A ¦ B; case ({S1, S0}) 2'b00: result = And; 2'b01: result = Or; 2'b10: result = fa_sum; 2'b11: result = fa_sum; default: result = 1'b0; endcase carry_out = fa_carry; end endmodule module _1bitALU_tb; reg A, B, Cin; reg b_inv; reg S0, S1; wire O, Cout; ALU_Behavioral myALU(A, B, Cin, O, Cout, S0, S1, b_inv); initial begin A = 0; B = 0; Cin = 0; b_inv = 0; S0 = 0; S1 = 0; #10; A = 1; B = 1; S0 = 0; S1 = 0; b_inv = 0; #10; A = 0; B = 1; S0 = 0; S1 = 1; b_inv = 0; #10; A = 1; B = 1; S0 = 0; S1 = 1; b_inv = 0; #10; A = 1; B = 1; Cin = 0; S0 = 1; S1 = 0; b_inv = 0; #10; A = 1; B = 1; Cin = 1; S0 = 1; S1 = 0; b_inv = 0; #10; A = 1; B = 0; Cin = 1; S0 = 1; S1 = 0; b_inv = 1; #10; A = 0; B = 1; Cin = 1; S0 = 1; S1 = 1; b_inv = 1; end endmodule Logisim:

Output:

Task 2:

Description:

Write Verilog code to build a 4bit ALU out of 4 1bit ALUs. The ALU must be capable of performing ADD, SUB operations and logical operation that are AND and OR operations and a SLT operation. Status flags (CF, SF, ZF and OF) must also be set accordingly Test the code using test bench and simulation.

Code:

module myAND( output O, input A, B, C, D ); wire t0, t1; and(t0, A, B); and(t1, C, D); and(O, t0, t1); endmodule module Multiplexer_GLevel( output O, input S0, S1, I0, I1, I2, I3 ); wire SN0, SN1; not(SN0, S0); not(SN1, S1); wire t0, t1, t2, t3; myAND A1(t0, I0, SN0, SN1, 1); myAND A2(t1, I1, SN0, S1, 1); myAND A3(t2, I2, S0, SN1, 1); myAND A4(t3, I3, S0, S1, 1); wire to0, to1; or(to0, t0, t1); or(to1, to0, t2); or(O, t3, to1); endmodule module HalfAdder_GLevel(Sum, Carry, A, B); input A, B; output Sum, Carry; xor sum(Sum, A, B); and carry(Carry, A, B); endmodule module FullAdder_GLevel(Sum, Carry, A, B, Cin); input A, B, Cin; output Sum, Carry; wire s1, c1, c2; HalfAdder_GLevel ha1(s1, c1, A, B); HalfAdder_GLevel ha2(Sum, c2, s1, Cin); or carry(Carry, c1, c2); endmodule // ----------- 1-BIT ALU MODULE ------------------ module ALU_1bit_GLevel(A, B, carry_in, result, carry_out, S0, S1, b_inv, less); input A, B, S0, S1, b_inv, carry_in, less; output result, carry_out; wire b_final; wire fa_sum; wire fa_carry; wire And; wire Or; xor (b_final, B, b_inv); FullAdder_GLevel fa1(fa_sum, fa_carry, A, b_final, carry_in); and a(And, A, B); or o(Or, A, B); // Multiplexer selects: // 00: AND, 01: OR, 10: ADD/SUB, 11: SLT Multiplexer_GLevel mux(result, S0, S1, And, Or, fa_sum, less); assign carry_out = fa_carry; endmodule // ----------- 4-BIT ALU MODULE ------------------ module ALU_4bit_GLevel( input [3:0] A, input [3:0] B, input S0, S1, b_inv, carry_in, output [3:0] result, output carry_out, output cf, sf, zf, of ); wire [4:0] c; wire [3:0] fa_results; wire slt_val; assign c[0] = carry_in ¦ b_inv; assign slt_val = sf ^ of; ALU_1bit_GLevel alu0(A[0], B[0], c[0], result[0], c[1], S0, S1, b_inv, slt_val); ALU_1bit_GLevel alu1(A[1], B[1], c[1], result[1], c[2], S0, S1, b_inv, 1'b0); ALU_1bit_GLevel alu2(A[2], B[2], c[2], result[2], c[3], S0, S1, b_inv, 1'b0); ALU_1bit_GLevel alu3(A[3], B[3], c[3], result[3], c[4], S0, S1, b_inv, 1'b0); assign carry_out = c[4]; assign cf = c[4] & (S0); // Carry Flag active during arithmetic assign sf = result[3]; // Sign Flag (MSB of result) assign zf = ~(¦result); // Zero Flag (1 if all bits are 0) assign of = c[4] ^ c[3]; // Overflow Flag (C4 ^ C3) endmodule // ----------- TESTBENCH ------------------ module ALU_4bit_tb; reg [3:0] A, B; reg S0, S1, b_inv, carry_in; wire [3:0] O; wire Cout, CF, SF, ZF, OF; ALU_4bit_GLevel myALU4(A, B, S0, S1, b_inv, carry_in, O, Cout, CF, SF, ZF, OF); initial begin A = 4'b1100; B = 4'b1010; S0 = 0; S1 = 0; b_inv = 0; carry_in = 0; //1100 & 1010 = 1000 #10; A = 4'b1100; B = 4'b1010; S0 = 1; S1 = 0; b_inv = 0; carry_in = 0; //1100 ¦ 1010 = 1110 #10; A = 4'b0010; B = 4'b0011; S0 = 0; S1 = 1; b_inv = 0; carry_in = 0; //0010 + 0011 = 0101 #10; A = 4'b0101; B = 4'b0010; S0 = 0; S1 = 1; b_inv = 1; carry_in = 1; //0101 - 0010 = 0011 #10; A = 4'b0010; B = 4'b0101; S0 = 1; S1 = 1; b_inv = 1; carry_in = 1; //0010 < 0101 -> true, so result = 0001 #10; A = 4'b0011; B = 4'b0011; S0 = 0; S1 = 1; b_inv = 1; carry_in = 1; //0011 - 0011 = 0000 -> ZF=1 end endmodule

Output:

Task 3:

Description:

Write a Verilog code for a parameterized ALU, where the parameter N specifies the bit-width of the input and output signals. The operation of the ALU is determined by a separate control signal. The ALU should support the following operations: a. Addition b. Subtraction c. Basic logical operations (AND, OR, NOT, XOR) The inputs and outputs of the ALU must be N-bit wide, making the design scalable and reusable for different bit sizes.

Code:

module Parameterized_ALU #(parameter N = 4) ( input [N-1:0] A, input [N-1:0] B, input [2:0] op, output reg [N-1:0] O, output reg Cout, output reg ZF, output reg SF, output reg OF ); always @(*) begin Cout = 0; OF = 0; case(op) 3'b000: {Cout, O} = A + B; 3'b001: {Cout, O} = A - B; 3'b010: O = A & B; 3'b011: O = A ¦ B; 3'b100: O = ~A; 3'b101: O = A ^ B; default: O = 0; endcase ZF = (O == 0); SF = O[N-1]; if (op == 3'b000) OF = (A[N-1] == B[N-1]) && (O[N-1] != A[N-1]); else if (op == 3'b001) OF = (A[N-1] != B[N-1]) && (O[N-1] != A[N-1]); end endmodule module ALU_4bit_tb; reg [3:0] A, B; reg [2:0] op; wire [3:0] O; wire Cout, ZF, SF, OF; Parameterized_ALU #(4) myALU4(A, B, op, O, Cout, ZF, SF, OF); initial begin A = 4'b1100; B = 4'b1010; op = 3'b010; // 1100 & 1010 = 1000 #10; A = 4'b1100; B = 4'b1010; op = 3'b011; // 1100 ¦ 1010 = 1110 #10; A = 4'b1100; B = 4'b1010; op = 3'b100; // ~1100 = 0011 #10; A = 4'b1100; B = 4'b1010; op = 3'b101; // 1100 ^ 1010 = 0110 #10; A = 4'b0010; B = 4'b0011; op = 3'b000; // 0010 + 0011 = 0101 #10; A = 4'b0101; B = 4'b0010; op = 3'b001; // 0101 - 0010 = 0011 end endmodule

Output:

Task 4:

Description:

Design and model a fully functional 32-bit ALU using Verilog. The ALU must perform a specified set of arithmetic, logical, and shifting operations (given in table 1) based on a set of control signals, and correctly manage status flags (given in table 2).

Code:

module alu_32bit ( input [31:0] a, input [31:0] b, input [3:0] alu_op, output reg [31:0] result, output zero ); always @(*) begin case (alu_op) 4'b0000: result = a + b; // ADD 4'b0001: result = a - b; // SUB 4'b0010: result = a & b; // AND 4'b0011: result = a ¦ b; // OR 4'b0100: result = a ^ b; // XOR 4'b0101: result = ($signed(a) < $signed(b)) ? 32'd1 : 32'd0; // SLT 4'b0110: result = (a < b) ? 32'd1 : 32'd0; // SLTU 4'b0111: result = a << b[4:0]; // SLL 4'b1000: result = a >> b[4:0]; // SRL 4'b1001: result = $signed(a) >>> b[4:0]; // SRA default: result = 32'd0; endcase end assign zero = (result == 32'd0); endmodule module ALU_32bit_tb; reg [31:0] A, B; reg [3:0] alu_op; wire [31:0] O; wire zero; alu_32bit myALU32(A, B, alu_op, O, zero); initial begin A = 32'd15; B = 32'd10; alu_op = 4'b0000; // 15 + 10 = 25 #10; A = 32'd15; B = 32'd10; alu_op = 4'b0001; // 15 - 10 = 5 #10; A = 32'hF0F0; B = 32'hFF00; alu_op = 4'b0010; // F0F0 & FF00 = F000 #10; A = 32'hF0F0; B = 32'h0F0F; alu_op = 4'b0011; // F0F0 ¦ 0F0F = FFFF #10; A = 32'hFFFF; B = 32'h0FF0; alu_op = 4'b0100; // FFFF ^ 0FF0 = F00F #10; A = -32'd5; B = 32'd3; alu_op = 4'b0101; // -5 < 3 -> 1 #10; A = 32'd4; B = 32'd2; alu_op = 4'b0111; // 4 << 2 = 16 #10; A = 32'd10; B = 32'd10; alu_op = 4'b0001; // 10 - 10 = 0 -> zero=1 end endmodule

Output:

Task 5: