Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#06)

Use of Venus Simulator for RISC-V Procedure Execution

Student Name: _Syed Sikandar Abbas_ Roll No: _24I-6553_ Section: CE-3B

Date performed: _September 30, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

In this task we were provided with a code that wasn’t working. The code was calling a procedure which was using a saved value from the caller procedure and it was overwriting it without the caller noticing. Because of this, the caller procedure was getting a different result. To solve this, the callee procedure was altered to use temporary registers to get its own task done. In this way, the saved value from the caller was preserved and the result was obtained as intended.

Code:

.text .globl main main: li a0, 1 li s0, 2500 jal ra, procedure # call procedure add t1, s0, a0 # result should be 1003+2500 li a0, 1 # setup system call for display (command in a0) mv a1, t1 # data to display in a1 ecall # displays result on console li a7, 93 li a0, 10 ecall procedure: li t2, 3 # Use temporary register t2 instead of s0 li t0, 1000 add a0, t2, t0 ret # return

Output:

Task 2:

Description:

In this task a piece of code was provided and we were supposed to explain it. It was storing 3 values in the stack and loading it. The step-by-step explanation is provided in the code comments.

Code:

addi s0, zero, 5 # s0 = 5 addi t1, zero, 10 # t1 = 10 addi t0, zero,15 # t0 = 15 addi sp, sp, -12 # allocates three bytes in stack sw s0, 0(sp) # save s0 (5) at stack memory address sp + 0 sw t0, 4(sp) # save t0 (15) at stack memory address sp + 4 sw t1, 8(sp) # save t1 (10) at stack memory address sp + 8 lw t4, 8(sp) # load value from sp + 8 (10) into t4 lw t2, 4(sp) # load value from sp + 4 (15) into t2 lw t3, 0(sp) # load value from sp + 0 (5) into t3 addi sp, sp, 12 # free the stack

Output:

Task 3:

Description:

In this task a C code snippet was given and we were required to produce its RISC-V assembly equivalent code. It used a procedure to solve an equation.

Code:

.data .text main: li a3, 5 li a4, 7 jal ra, EqSolver j end EqSolver: add t0, a3, a4 sub t1, a3, a4 mul a0, t0, t1 ret end:

Output:

Task 4:

Description:

In this task we were asked to write a RISC-V assembly code that has two procedures, AreaFinder and PerimeterFinder which take the side and calculate the Area and Perimeter of a given square.

Code:

.data .text .globl main main: li a0, 4 jal ra, AreaFinder mv s0, a0 # store a0 in s0. s0 is area jal ra, PerimeterFinder # a0 now has the perimeter mv s1, a0 j end AreaFinder: mul a0, a0, a0 ret PerimeterFinder: li t0, 4 mul a0, a0, t0 ret end:

Output:

Task 5:

Description:

In this code, a C snippet was provided and we were supposed to write its RISC-V assembly equivalent code that takes a value of x and produces the value of a function at x. The polynomial was defined as x^3 + 5x^2 + 6x.

Code:

.text main: li a3, 5 jal ra, EqSolver mv s0, a0 j end EqSolver: # x^3+5x^2+6x mul t0, a3, a3 mul t0, t0, a3 addi t5, x0, 5 addi t6, x0, 6 mul t1, a3, a3 mul t1, t1, t5 mul t2, t6, a3 add t3, t0, t1 add a0, t3, t2 end:

Output:

Task 6:

Description:

This task required us to write a RISC-V assembly program from a given C program. It had multiple proceduers that were to be executed in case of different values of n and k.

Code:

.text main: li a0, 5 # n li a1, 7 # k li s0, 1 beq a1, s0, callF1 bgt a0, a1, callF2 jal ra, F3 j exit callF1: jal ra, F1 j exit callF2: jal ra, F2 j exit F1: mul t0, a0, a0 # x2 mul t1, a1, a1 # z2 add a0, t0, t1 ret F2: mul t0, a0, a0 # a^2 mul a0, t0, a0 # a^2 * a = a^3 ret F3: addi a0, a0, 5 ret exit:

Output:

Task 7:

Description:

This task required us to write a RISC-V assembly program from a given C program. It had multiple proceduers that were to be executed in case of different values of n and k.

Code:

.text .globl main main: li a0, 5 #n li a1, 9 #k beq a1, zero, callfun1 beq a0, a1, callfun2 jal ra, fun3 j exit callfun1: jal ra, fun1 j exit callfun2: jal ra, fun2 j exit fun1: mul a0, a0, a0 ret fun2: mul t0, a0, a0 mul a0, t0, a0 ret fun3: addi a0, a0, 1 ret exit:

Output:

Task 8:

| Description: |  |
| --- | --- |
| In this task, we were required to translate two C codes to RISC-V assembly codes to find the Fibonacci series of a number ‘n’. One code was Without recursion, using a for loop and the other one was through recursion. |  |
| Code: (Without Recursion) (With Recursion) |  |
| .text .globl main main: li a0, 10 #n jal ra, fib j exit fib: li t0, 0 # a li t1, 1 # b li t2, 0 # c li t3, 2 # i bne a0, x0, Loopstart li a0, 1 ret Loopstart: bgt t3, a0, Loopend add t2, t0, t1 mv t0, t1 mv t1, t2 addi t3, t3, 1 j Loopstart Loopend: ret exit: | .text .globl main main: li a0, 10 jal ra, fib j exit fib: beq a0, zero, ifzero li t0, 1 beq a0, t0, ifone addi sp, sp, -16 sw ra, 12(sp) sw s0, 8(sp) sw s1, 4(sp) mv s0, a0 addi a0, s0, -1 jal ra, fib mv s1, a0 addi a0, s0, -2 jal ra, fib add a0, s1, a0 lw s1, 4(sp) lw s0, 8(sp) lw ra, 12(sp) addi sp, sp, 16 ret ifzero: li a0, 0 ret ifone: li a0, 1 ret exit: li a0, 10 ecall |
| Output: |  |
| Without Recursion With Recursion |  |