Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#09)

Implementation of a Register File using ModelSim

Student Name: _______Syed Sikandar Abbas___ Roll No: __i246553___ Section: CE-B

Date performed: _____October 21, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

Verilog code for 4x4 register file.

Code:

module decoder_2_to4 (output reg [3:0] dec_out, input [1:0] wr, input write_enable); always @(*) begin if (write_enable) begin case (wr) 2'b00: dec_out = 4'b0001; 2'b01: dec_out = 4'b0010; 2'b10: dec_out = 4'b0100; 2'b11: dec_out = 4'b1000; default: dec_out = 4'b0000; endcase end else begin dec_out = 4'b0000; end end endmodule module mux4_1 (output reg [3:0] out, input [3:0] in, in1, in2, in3, input [1:0] se1); always @(*) begin if (se1 == 2'b00) out = in; else if (se1 == 2'b01) out = in1; else if (se1 == 2'b10) out = in2; else if (se1 == 2'b11) out = in3; else out = 4'b0000; // optional, for safety end endmodule module register_4bit (output reg [3:0] q, input [3:0] d, input load, input clk); initial begin q <= 4'b0000; end always @(posedge clk) begin if (load) q <= d; end endmodule module register_file_4x4 (data_out1, data_out2, data_in, wr, write_enable, rd1, rd2, clk); // output ports output [3:0] data_out1; output [3:0] data_out2; // input ports input [3:0] data_in; input [1:0] wr; input write_enable; input [1:0] rd1, rd2; input clk; // Internal wires wire [3:0] dec_out; wire [3:0] q0, q1, q2, q3; // Instantiate decoder decoder_2_to4 decoder (dec_out, wr, write_enable); // Instantiate four 4-bit registers register_4bit reg0 (q0, data_in, dec_out [0], clk); register_4bit reg1 (q1, data_in, dec_out [1], clk); register_4bit reg2 (q2, data_in, dec_out [2], clk); register_4bit reg3(q3, data_in, dec_out [3], clk); // Instantiate two 4-to-1 multiplexers for reading mux4_1 muxl (data_outl, q0, q1, q2, q3, rdl); mux4_1 mux2 (data_out2, q0, q1, q2, q3, rd2); endmodule

Task 2:

Description:

Verilog Code & Testbench for 4x4 Register file implemented in Task 1.

Code:

module decoder_2_to4 (output reg [3:0] dec_out, input [1:0] wr, input write_enable); always @(*) begin if (write_enable) begin case (wr) 2'b00: dec_out = 4'b0001; 2'b01: dec_out = 4'b0010; 2'b10: dec_out = 4'b0100; 2'b11: dec_out = 4'b1000; default: dec_out = 4'b0000; endcase end else begin dec_out = 4'b0000; end end endmodule module mux4_1 (output reg [3:0] out, input [3:0] in, in1, in2, in3, input [1:0] se1); always @(*) begin if (se1 == 2'b00) out = in; else if (se1 == 2'b01) out = in1; else if (se1 == 2'b10) out = in2; else if (se1 == 2'b11) out = in3; else out = 4'b0000; // optional, for safety end endmodule module register_4bit (output reg [3:0] q, input [3:0] d, input load, input clk); initial begin q <= 4'b0000; end always @(posedge clk) begin if (load) q <= d; end endmodule module register_file_4x4 (data_out1, data_out2, data_in, wr, write_enable, rd1, rd2, clk); // output ports output [3:0] data_out1; output [3:0] data_out2; // input ports input [3:0] data_in; input [1:0] wr; input write_enable; input [1:0] rd1, rd2; input clk; // Internal wires wire [3:0] dec_out; wire [3:0] q0, q1, q2, q3; // Instantiate decoder decoder_2_to4 decoder (dec_out, wr, write_enable); // Instantiate four 4-bit registers register_4bit reg0 (q0, data_in, dec_out [0], clk); register_4bit reg1 (q1, data_in, dec_out [1], clk); register_4bit reg2 (q2, data_in, dec_out [2], clk); register_4bit reg3(q3, data_in, dec_out [3], clk); // Instantiate two 4-to-1 multiplexers for reading mux4_1 muxl (data_outl, q0, q1, q2, q3, rdl); mux4_1 mux2 (data_out2, q0, q1, q2, q3, rd2); endmodule module tb_4bit_RFile(); reg [3:0] d_in; reg [1:0] wr; reg write_en; reg [1:0] rd1, rd2; reg clk; wire [3:0] out1; wire [3:0] out2; register_file_4x4 RFile( .data_out1(out1), .data_out2(out2), .data_in(d_in), .wr(wr), .write_enable(write_en), .rd1(rd1), .rd2(rd2), .clk(clk) ); always begin #5 clk = ~clk; end initial begin clk = 0; write_en = 0; d_in = 0'b0; wr = 2'b0; rd1 = 2'b00; rd2 = 2'b01; #10 write_en = 1; // i) ------ R1 <- 6 & R2 <- 3 d_in = 4'b0110; wr = 2'b01; #10 d_in = 4'b0011; wr = 2'b10; #10 // ii) ------ R3 <- 13 d_in = 4'b1101; wr = 2'b11; #10 // iii) ----- out1 <- R1 & out2 <- R2 write_en = 0; rd1 = 2'b01; rd2 = 2'b10; end endmodule

Output:

Task 3:

Description:

A 32-bit Register File for a RISC-V (RV32) processor architecture was designed, implemented and verified using Verilog.

Code:

module decoder_5_to_32 ( output reg [31:0] dec_out, input [4:0] wr, input write_enable ); always @(*) begin if (write_enable && (wr != 5'b00000)) begin dec_out = 32'b1 << wr; end else begin dec_out = 32'h00000000; end end endmodule module register_32bit ( output reg [31:0] q, input [31:0] d, input load, input clk, input rst ); always @(posedge clk or posedge rst) begin if (rst) q <= 32'h00000000; else if (load) q <= d; end endmodule module mux32_1 ( output reg [31:0] out, input [31:0] in0, in1, in2, in3, in4, in5, in6, in7, in8, in9, in10, in11, in12, in13, in14, in15, in16, in17, in18, in19, in20, in21, in22, in23, in24, in25, in26, in27, in28, in29, in30, in31, input [4:0] sel ); always @(*) begin case (sel) 5'd0: out = 32'h00000000; //x0 is hardwired to 0 5'd1: out = in1; 5'd2: out = in2; 5'd3: out = in3; 5'd4: out = in4; 5'd5: out = in5; 5'd6: out = in6; 5'd7: out = in7; 5'd8: out = in8; 5'd9: out = in9; 5'd10: out = in10; 5'd11: out = in11; 5'd12: out = in12; 5'd13: out = in13; 5'd14: out = in14; 5'd15: out = in15; 5'd16: out = in16; 5'd17: out = in17; 5'd18: out = in18; 5'd19: out = in19; 5'd20: out = in20; 5'd21: out = in21; 5'd22: out = in22; 5'd23: out = in23; 5'd24: out = in24; 5'd25: out = in25; 5'd26: out = in26; 5'd27: out = in27; 5'd28: out = in28; 5'd29: out = in29; 5'd30: out = in30; 5'd31: out = in31; default: out = 32'h00000000; endcase end endmodule module register_file_32x32 ( output [31:0] rdata1, output [31:0] rdata2, input [31:0] wdata, input [4:0] rd, input [4:0] rs1, input [4:0] rs2, input reg_write, input clk, input rst ); wire [31:0] dec_out; wire [31:0] q0, q1, q2, q3, q4, q5, q6, q7, q8, q9, q10, q11, q12, q13, q14, q15, q16, q17, q18, q19, q20, q21, q22, q23, q24, q25, q26, q27, q28, q29, q30, q31; assign q0 = 32'h00000000; decoder_5_to_32 decoder (dec_out, rd, reg_write); register_32bit reg1 (q1, wdata, dec_out[1], clk, rst); register_32bit reg2 (q2, wdata, dec_out[2], clk, rst); register_32bit reg3 (q3, wdata, dec_out[3], clk, rst); register_32bit reg4 (q4, wdata, dec_out[4], clk, rst); register_32bit reg5 (q5, wdata, dec_out[5], clk, rst); register_32bit reg6 (q6, wdata, dec_out[6], clk, rst); register_32bit reg7 (q7, wdata, dec_out[7], clk, rst); register_32bit reg8 (q8, wdata, dec_out[8], clk, rst); register_32bit reg9 (q9, wdata, dec_out[9], clk, rst); register_32bit reg10 (q10, wdata, dec_out[10], clk, rst); register_32bit reg11 (q11, wdata, dec_out[11], clk, rst); register_32bit reg12 (q12, wdata, dec_out[12], clk, rst); register_32bit reg13 (q13, wdata, dec_out[13], clk, rst); register_32bit reg14 (q14, wdata, dec_out[14], clk, rst); register_32bit reg15 (q15, wdata, dec_out[15], clk, rst); register_32bit reg16 (q16, wdata, dec_out[16], clk, rst); register_32bit reg17 (q17, wdata, dec_out[17], clk, rst); register_32bit reg18 (q18, wdata, dec_out[18], clk, rst); register_32bit reg19 (q19, wdata, dec_out[19], clk, rst); register_32bit reg20 (q20, wdata, dec_out[20], clk, rst); register_32bit reg21 (q21, wdata, dec_out[21], clk, rst); register_32bit reg22 (q22, wdata, dec_out[22], clk, rst); register_32bit reg23 (q23, wdata, dec_out[23], clk, rst); register_32bit reg24 (q24, wdata, dec_out[24], clk, rst); register_32bit reg25 (q25, wdata, dec_out[25], clk, rst); register_32bit reg26 (q26, wdata, dec_out[26], clk, rst); register_32bit reg27 (q27, wdata, dec_out[27], clk, rst); register_32bit reg28 (q28, wdata, dec_out[28], clk, rst); register_32bit reg29 (q29, wdata, dec_out[29], clk, rst); register_32bit reg30 (q30, wdata, dec_out[30], clk, rst); register_32bit reg31 (q31, wdata, dec_out[31], clk, rst); mux32_1 mux1 (rdata1, q0, q1, q2, q3, q4, q5, q6, q7, q8, q9, q10, q11, q12, q13, q14, q15, q16, q17, q18, q19, q20, q21, q22, q23, q24, q25, q26, q27, q28, q29, q30, q31, rs1); mux32_1 mux2 (rdata2, q0, q1, q2, q3, q4, q5, q6, q7, q8, q9, q10, q11, q12, q13, q14, q15, q16, q17, q18, q19, q20, q21, q22, q23, q24, q25, q26, q27, q28, q29, q30, q31, rs2); endmodule module tb_register_file_32x32(); reg [31:0] d_in; reg [4:0] wr; reg write_en; reg [4:0] rd1, rd2; reg clk; reg rst; wire [31:0] out1; wire [31:0] out2; // Instantiate the 32x32 RISC-V Register File register_file_32x32 RFile( .rdata1(out1), .rdata2(out2), .wdata(d_in), .rd(wr), .rs1(rd1), .rs2(rd2), .reg_write(write_en), .clk(clk), .rst(rst) ); always begin #5 clk = ~clk; end initial begin clk = 0; rst = 1; write_en = 0; d_in = 32'b0; wr = 5'b00000; rd1 = 5'b00000; rd2 = 5'b00000; #10; rst = 0; #10; write_en = 1; d_in = 32'hABCDEFAB; wr = 5'b00000; #10; d_in = 32'd6; wr = 5'b00001; // register x1 #10; d_in = 32'd3; wr = 5'b00010; // register x2 #10; d_in = 32'd13; wr = 5'b00011; // register x3 #10; write_en = 0; rd1 = 5'b00001; // select x1 for read port 1 rd2 = 5'b00010; // select x2 for read port 2 #10; rd1 = 5'b00000; // select x0 (should be 0) rd2 = 5'b00011; // select x3 (should be 13) #10; end endmodule

Output:

Task 4:

Description:

Simulated a 32-bit Register in Logisim.

Circuit: